Current & Charge Calculations Worksheet

I = Q		Units: I in A (amperes)

t		        Q in C (coulombs)

t  in s (seconds)

current = charge moving past a point

time

Basic: Find the unknown quantity – show your working in your books:

| a) I = Q = 8 C t = 20 s | b) I = Q = 240 C t = 300 s | c) I = Q = 400 C t = 200 s | d) I = Q = 750 C t = 350 s | e) I = Q = 300 C t = 100 s | f) I = Q = 50 C t = 2 s |
| --- | --- | --- | --- | --- | --- |
| g) Re-arrange the equation for Q | h) I = 2.5 A Q = t = 300 s | i) I = 5 A Q = t = 200 s | j) I = 13 A Q = t = 350 s | k) I = 10 A Q = t = 100 s | l) I = 6 A Q = t = 2 s |
| m) Re-arrange the equation for t | n) I = 4 A Q = 240 C t = | o) I = 20 A Q = 400 C t = | p) I = 5 A Q = 750 C t = | q) I = 6 A Q = 300 C t = | r) I = 2.4 A Q = 50 C t = |

Medium: Find the unknown quantity (CONVERT FIRST to SECONDS)

| a) I = Q = 140 C t = 4 min = _______ s | b) I = 0.3 A Q = t = 1.5 hours = _______ s | c) I = 0.9 A Q = t = 3 min = _________ s |
| --- | --- | --- |
| d) I = Q = 200 C t = 5 min = _______ s | e) I = 1.5 A Q = t = 2 hours = _______ s | f) I = 0.4 A Q = t = 7 min = _________ s |

Hard: WORD PROBLEMS

- How much current must there be in a circuit if 1000 coulombs flow past a point in the circuit in 4 minutes?

- A circuit is switched on for half a minute and 90 coulombs of charge flowed.  What was the current flowing through the circuit?

- If there is a current of 10 mA in a circuit for 0.5 s, what quantity of electric charge flows in through the circuit?

- How much time is required for 0.3 coulombs of charge to flow past a point if the rate of flow (current) is 2 mA?

- During electrolysis 6A was passed through some copper chloride and a charge of 1.2 kC flowed.  How long was the experiment on for?

- A bed lamp is switched on for 10 minutes.  It works on a current of 0.5 A.  How much charge flowed?

Current & Charge Calculations Worksheet

I = Q		Units: I is A (amperes)

t		        Q is C (coulombs)

t  is s (seconds)

current = charge moving past a point

time

Basic: Find the unknown quantity – show your working in your books:

| a) I = Q = 8 C t = 20 s | b) I = Q = 240 C t = 300 s | c) I = Q = 400 C t = 200 s | d) I = Q = 750 C t = 350 s | e) I = Q = 300 C t = 100 s | f) I = Q = 50 C t = 2 s |
| --- | --- | --- | --- | --- | --- |
| g) Re-arrange the equation for Q | h) I = 2.5 A Q = t = 300 s | i) I = 5 A Q = t = 200 s | j) I = 13 A Q = t = 350 s | k) I = 10 A Q = t = 100 s | l) I = 6 A Q = t = 2 s |
| m) Re-arrange the equation for t | n) I = 4 A Q = 240 C t = | o) I = 20 A Q = 400 C t = | p) I = 5 A Q = 750 C t = | q) I = 6 A Q = 300 C t = | r) I = 2.4 A Q = 50 C t = |

Medium: Find the unknown quantity (CONVERT FIRST to SECONDS)

| a) I = Q = 140 C t = 4 min = _______ s | b) I = 0.3 A Q = t = 1.5 hours = _______ s | c) I = 0.9 A Q = t = 3 min = _________ s |
| --- | --- | --- |
| d) I = Q = 200 C t = 5 min = _______ s | e) I = 1.5 A Q = t = 2 hours = _______ s | f) I = 0.4 A Q = t = 7 min = _________ s |

Hard: WORD PROBLEMS

- How much current must there be in a circuit if 1000 coulombs flow past a point in the circuit in 4 minutes?

- A circuit is switched on for half a minute and 90 coulombs of charge flowed.  What was the current flowing through the circuit?

- If there is a current of 10 mA in a circuit for 0.5 s, what quantity of electric charge flows in through the circuit?

- How much time is required for 0.3 coulombs of charge to flow past a point if the rate of flow (current) is 2 mA?

- During electrolysis 6A was passed through some copper chloride and a charge of 1.2 kC flowed.  How long was the experiment on for?

- A bed lamp is switched on for 10 minutes.  It works on a current of 0.5 A.  How much charge flowed?