A circuit was set up as shown in the diagram.

(a)     The table gives the current through three of the ammeters. Complete the table to show the current through the other two ammeters.

| Ammeter | Reading on ammeter in amps |
| --- | --- |
| Al | 0.2 |
| A2 | 0.6 |
| A3 | 0.3 |
| A4 |  |
| A5 |  |

(2)

(b)     The potential difference across R2 is 12 V.

What is the resistance of R2?

Show your working and include the correct unit.

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

Resistance = ____________________

(3)

(c)     In the circuit above, the resistor R2 burned out and current stopped flowing in it. There was no other change to the circuit.

Complete the table below to show the readings on the ammeters after this took place.

| Ammeter | Reading on ammeter in amps |
| --- | --- |
| Al | 0.2 |
| A2 | 0.0 |
| A3 |  |
| A4 |  |
| A5 |  |

(3)

(Total 8 marks)

Mark schemes

Q1.

(a)     0.9

1

1.1

accept the value of A4 + 0.2

1

(b)     V = I R or 12 = 0.6 R or

accept V = A R
V = I × ohm’s sign
do not credit Ohm’s law triangle

2

R = 20

correct numerical answer earns both marks

ohms

1

(c)     A3 = 0.3

A4 = 0.3

accept the same numeric value as A3

A5 = 0.5

accept the value of A4 + 0.2

3

[8]