(a)     Some people wear magnetic bracelets to relieve pain.

Figure 1 shows a magnetic bracelet.

There are magnetic poles at both A and B.
Part of the magnetic field pattern between A and B is shown.

Figure 1

What is the pole at A? ______________________

What is the pole at B? ______________________

(1)

(b)     Figure 2 shows two of the lines of the magnetic field pattern of a current-carrying wire.

Figure 2

The direction of the current is reversed.

What happens to the direction of the lines in the magnetic field pattern?

___________________________________________________________________

(1)

(c)     Fleming’s left-hand rule can be used to identify the direction of a force acting on a current-carrying wire in a magnetic field.

(i)      Complete the labels in Figure 3.

Figure 3

(2)

(ii)     Figure 4 shows:

•        the direction of the magnetic field between a pair of magnets

•        the direction of the current in a wire in the magnetic field.

Figure 4

In which direction does the force on the wire act?

______________________________________________________________

(1)

(iii)    Suggest three changes that would decrease the force acting on the wire.

1. ____________________________________________________________

2. ____________________________________________________________

3. ____________________________________________________________

(3)

(d)     Figure 5 shows part of a moving-coil ammeter as drawn by a student.

The ammeter consists of a coil placed in a uniform magnetic field.
When there is a current in the coil, the force acting on the coil causes the coil to rotate and the pointer moves across the scale.

Figure 5

(i)      The equipment has not been set up correctly.

What change would make it work?

______________________________________________________________

______________________________________________________________

(1)

(ii)     Figure 6 shows the pointer in an ammeter when there is no current.

Figure 6

What type of error does the ammeter have?

______________________________________________________________

(1)

(Total 10 marks)

Mark schemes

Q1.

(a)     north (pole)

accept N

north (pole)

both needed for mark

1

(b)     reverses

accept changes direction

1

(c)     (i)      first finger:
(direction of) (magnetic) field

1

second finger:
(direction of) (conventional) current

1

(ii)     into (plane of the) paper

1

(iii)    less current in wire

accept less current / voltage / more resistance / thinner wire

1

weaker field

allow weaker magnets / magnets further apart

do not accept smaller magnets

1

rotation of magnets (so) field is no longer perpendicular to wire

1

(d)     (i)      reverse one of the magnets

do not accept there are no numbers on the scale

1

(ii)     systematic or zero error

accept all current values will be too big

accept it does not return to zero
accept it does not start at zero

1

[10]