Q1.

Scientists have used a satellite system to investigate the idea of generating electricity in space.

As the system orbited the Earth a 20 km copper wire was reeled out.

Before the wire snapped a current of 1 amp was induced in the wire.

Figure 1

(a)     What provides the force needed to keep a satellite in orbit around the Earth?

___________________________________________________________________

(1)

(b)     Explain how a current is induced in the wire.

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

(3)

An alternator is connected to a data logger.

The data logger is connected to a computer.

Figure 2 shows how the output potential difference of the alternator varies with time.

Figure 2

(c)     The coil inside the alternator now rotates at twice the frequency.

Draw on Figure 2 to show how the output potential difference varies with time at this new frequency.

(2)

Another type of generator is now connected to the data logger and computer.

Figure 3 shows how the output potential difference varies with time for this generator.

Figure 3

(d)     What name is given to this second type of generator?

___________________________________________________________________

(1)

(e)     Look at Figure 2 and Figure 3.

Give one difference between the outputs from the two types of generator.

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

(1)

(f)      The charger used to charge the battery inside a laptop computer contains a small transformer.

The charger plugs into the mains electricity supply.

mains electricity supply = 230 V

number of turns on the primary coil of the transformer = 690

number of turns on the secondary coil of the transformer = 57

Calculate the potential difference applied by the charger across the battery inside the computer.

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

Potential difference = ____________________ V

(3)

(Total 11 marks)

Mark schemes

Q1.

(a)     gravity

1

(b)     as the wire moves through the Earth’s magnetic field

1

a potential difference is induced between the ends of the wire

1

the wire must be part of a complete circuit

1

(c)     new trace shows:

twice the frequency

1

twice the amplitude

1

(d)     dynamo

dc generator is insufficient

1

(e)     the alternator pd changes polarity, the 2nd type of generator does not

1

(f)

1

1

Vs = 19 (V)

an answer of 19 (V) scores 3 marks

1

[11]