How electromagnets work (Loud Speaker) – Extended Response

The figure below shows the parts of a moving-coil loudspeaker.

A coil of wire is positioned in the gap between the north and south poles of the cylindrical magnet.

Explain how the loudspeaker converts current in an electrical circuit to a sound wave.

| Describe the current passing through the circuit. Complete? | ………………………………………………………………………….... ………………………………………………………………………….... ………………………………………………………………………….... ………………………………………………………………………….... ………………………………………………………………………….... ………………………………………………………………………….... ………………………………………………………………………….... ………………………………………………………………………….... ………………………………………………………………………….... ………………………………………………………………………….... ………………………………………………………………………….... ………………………………………………………………………….... ………………………………………………………………………….... ………………………………………………………………………….... ………………………………………………………………………….... ………………………………………………………………………….... ………………………………………………………………………….... |
| --- | --- |
| What effect does the current have on the coil? Complete? |  |
| What effect does the coil have on the cone? Complete? |  |
| What effect does the cone have on the air molecules? Complete? |  |

How electromagnets work (Loud Speaker) – Mark scheme

Level 3 (5–6 marks):

A detailed explanation linking variations in current to the pressure variations of a sound wave, with a logical sequence.

Level 2 (3–4 marks):

A number of relevant points made, but not precisely. A link between the loudspeaker and
a sound wave is made.

Level 1 (1–2 marks):

Some relevant points but fragmented with no logical structure.

0 marks:

No relevant content.

Indicative content

the current in the electrical circuit is varying

the current passes through the coil

the coil experiences a force (inwards or outwards)

reversing the current reverses the force

the size of the current affects the size of the force

the varying current causes the coil to vibrate

the (vibrating) coil causes the cone to vibrate

the vibrating cone causes the air molecules to move

the movement of the air molecules produces the pressure variations in the air needed for
a sound wave

the air molecules bunch together forming compressions and spread apart forming rarefactions

6