The National Grid is a network of cables, pylons and transformers that distributes (‘shares out’) electrical energy across the UK.

The diagram shows a simplified model of the National Grid system.

1. Which letter represents the power station? __________

2. Which letters represent a pylon? ________ and ________

3. Which letter represents the cables of the National Grid? ________

4. Which letter represents a house close to the power station? ________

5. Which letter represents a house far away from the power station? ________

6. Bulb B is brighter than / the same brightness as / dimmer than bulb A.

7. This happens because energy is gained / remains constant / is lost as electric current travels through the cable because of resistance.

8. The wasted / gained energy results in heating / cooling of the cables.

9. This system can be described as efficient / inefficient because some energy is wasted instead of being transferred usefully.

10. Which Sankey Diagram best represents the energy transfer to bulb B in this model?

This diagram shows a more realistic model of the National Grid system.

11. Name the two items that have been added. ________________________

12. What is the letter of the step up transformer? ________

13. What is the letter of the step down transformer? ________

14. Bulb B is brighter than / has a similar brightness to / dimmer than bulb A.

15. This indicates that more / less energy is being wasted compared with the simple model.

16. (a) This indicates that the system is more / less efficient than before.

(b) Which Sankey Diagram could show the energy transfer to Bulb B now?

17. The step up transformer increases / decreases the voltage of the power supply.

18. This results in the current being stepped down / stepped up.

19. The same amount of energy is being transferred as before: however, it is being transmitted as a high / low voltage rather than a high / low current.

20. More / The same / Less energy is lost because the energy is carried as a large voltage rather than as a large current.

21. The step down transformer increases / decreases the voltage to a safer level before it is used by the consumer.

The National Grid is a network of cables, pylons and transformers that distributes (‘shares out’) electrical energy across the UK.

The diagram shows a simplified model of the National Grid system.

1. Which letter represents the power station? __________

2. Which letters represent a pylon? ________ and ________

3. Which letter represents the cables of the National Grid? ________

4. Which letter represents a house close to the power station? ________

5. Which letter represents a house far away from the power station? ________

6. Bulb B is brighter than / the same brightness as / dimmer than bulb A.

7. This happens because energy is gained / remains constant / is lost as electric current travels through the cable because of resistance.

8. The wasted / gained energy results in heating / cooling of the cables.

9. This system can be described as efficient / inefficient because some energy is wasted instead of being transferred usefully.

10. Which Sankey Diagram best represents the energy transfer to bulb B in this model?

This diagram shows a more realistic model of the National Grid system.

11. Name the two items that have been added. ________________________

12. What is the letter of the step up transformer? ________

13. What is the letter of the step down transformer? ________

14. Bulb B is brighter than / has a similar brightness to / dimmer than bulb A.

15. This indicates that more / less energy is being wasted compared with the simple model.

16. (a) This indicates that the system is more / less efficient than before.

(b) Which Sankey Diagram could show the energy transfer to Bulb B now?

17. The step up transformer increases / decreases the voltage of the power supply.

18. This results in the current being stepped down / stepped up.

19. The same amount of energy is being transferred as before: however, it is being transmitted as a high / low voltage rather than a high / low current.

20. More / The same / Less energy is lost because the energy is carried as a large voltage rather than as a large current.

21. The step down transformer increases / decreases the voltage to a safer level before it is used by the consumer.