Basic

- A 1.5 V cell is connected to a 3.0 Ω resistor and a 2.0 Ω resistor in series with each other.

- Draw a circuit diagram for this circuit

- Calculate:

- The total resistance of the two resistors

- The current through the resistors

- The potential differences across each resistor

Medium

- A circuit contains a battery of two cells, with each cell providing 1.5 V. The circuit also has two resistors connected in series. Resistor P has a resistance of 2 Ω and resistor Q has a resistance of 10 Ω.

- Draw a circuit diagram for this circuit.

- Calculate the total resistance of the two resistors

- Calculate the total potential difference provided by the battery

- Show that the current through the battery is 0.25 A

- Calculate the potential difference across each resistor

- A circuit contains a 6 V battery and three resistors connected in parallel with each other and with the battery.

R1 = 2 Ω

R2 = 3 Ω

R3 = 6 Ω

- Draw a circuit diagram for this circuit

- Calculate the current through each resistor

- Calculate the current through the battery

Hard

A1

A3

A2

The battery in this circuit has a potential difference of 12V, each bulb has a resistance of 5Ω, calculate the current in A1, A2 and A3

- The two bulbs are identical, calculate the voltage over them

- What would happen to the brightness of the bulbs if you added another bulb in series with the first two?

- What would happen to the brightness of the bulbs if you added another two bulbs in parallel to the first two?