A 12 V filament bulb is connected to a 12 V power supply.
The graph shows how the current changes after the bulb is switched on.

(a)     (i)      After 0.10 seconds, the bulb works at its normal brightness.

What is the current through the bulb when it is working at normal brightness?

Current = _____________________ A

(1)

(ii)     The bulb works at normal brightness for 30 seconds before it is switched off.

Calculate the charge that flows through the bulb in the 30 seconds before it is switched off. Give the unit.

______________________________________________________________

______________________________________________________________

______________________________________________________________

Charge = _________________ unit _________________

(3)

(iii)    Calculate the energy transferred by the 12 V bulb when it is working at normal brightness for 30 seconds.

______________________________________________________________

______________________________________________________________

Energy transferred = _____________________ J

(2)

Mark schemes

Q1.

(a)     (i)      1.7

1

(ii)     51
or
30 × their (i) correctly calculated

allow 1 mark for correct substitution i.e. 1.7

or their (i)

2

coulomb / C

do not accept c

1

(iii)    612
or
their (ii) × 12 correctly calculated
or
their (i) × 360 correctly calculated

allow 1 mark for correct substitution i.e. E = 12 × 51

or 12 × their (ii)

or their (i) × 360

2

(b)     ions vibrate faster
or
ions vibrate with a bigger amplitude

accept atoms for ions throughout

accept ions gain energy

accept ions vibrate more

ions start to vibrate is insufficient

1

electrons collide more (frequently) with the ions
or
(drift) velocity of electrons decreases

electrons start to collide is insufficient

there are more collisions is insufficient, unless both electrons and ions are implied

1

[8]