The diagram shows a temperature sensing circuit used to control a heating system in a house.

(a)     What quantity does the ammeter measure?

___________________________________________________________________

(1)

(b)     The current in the circuit is 3.5 mA when the potential difference across the thermistor is 4.2 V

Calculate the resistance of the thermistor.

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

Resistance = ____________________ Ω

(3)

(c)     Calculate the charge that flows through the thermistor in 5 minutes when the current is 3.5 mA.

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

Charge = ____________________ C

(3)

(d)     Explain why the potential difference across the thermistor changes as the temperature in the house decreases.

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

(2)

(e)     The circuit shown in the diagram can be modified to turn lights on and off by replacing the thermistor with a Light Dependent Resistor (LDR).

Draw the circuit symbol for an LDR in the space below.

(1)

(Total 10 marks)

Mark schemes

Q1.

(a)     current

1

(b)     4.2 = 3.5 × 10–3 × R

1

R = 4.2 / 3.5 × 10–3

1

R = 1200 (Ω)

an answer of 1200 (Ω) scores 3 marks

an answer of 1.2 scores 2 marks

1

(c)     conversion from minutes to seconds (300 s)

1

Q = 0.0035 × (5 × 60)

1

Q = 1.05 C

an answer of 1.05 (C) scores 3 marks

an answer of 17.5 scores 1 mark

an answer of 1050 or 0.0175 scores 2 marks

1

(d)     (potential difference) increases

1

(because thermistor) resistance increases

2nd mark dependent on scoring 1st mark

1

(e)

1

[10]