Q1.

(a)    In the National Grid, very large step-up transformers link power stations to the transmission cables.

A transformer used for this purpose has 800 turns on its primary coil and 12 800 turns on its secondary coil. The p.d. (potential difference) across its primary coil is 25 kV.

Use the equation in the box to calculate the p.d. across its secondary coil.

Show clearly how you work out your answer.

___________________________________________________________________

___________________________________________________________________

p.d. across secondary coil = _________________________ volts

(2)

(b)     The diagram shows the structure of a transformer.

(i)      The primary and secondary coils of a transformer are made of insulated wire.

Why is this insulation necessary?

______________________________________________________________

______________________________________________________________

(1)

(ii)     Why is the core made of iron?

______________________________________________________________

______________________________________________________________

(1)

(iii)    Explain how the transformer works.

______________________________________________________________

______________________________________________________________

______________________________________________________________

______________________________________________________________

______________________________________________________________

______________________________________________________________

(3)

(c)     Before 1926, large towns had their own local power stations. After 1926, these power stations were connected to form the National Grid.

Give two advantages of having a National Grid system.

1. _________________________________________________________________

___________________________________________________________________

2. _________________________________________________________________

___________________________________________________________________

(2)

(Total 9 marks)

Mark schemes

Q1.

(a)    400 000

allow 1 mark for correct substitution ie

or

2

(b)     (i)      any one from:

do not accept any response in terms of heat insulation, safety or electric shock

•         (so that there is) no short circuit

•         (so that the) current goes around the coil

do not accept electricity for current

•         (so that the) current does not enter the core

1

(ii)     (easily) magnetised (and demagnetised)

accept ‘(it’s) magnetic’ 
do not accept ‘because it’s a conductor’

1

(iii)    alternating current in the primary (coil)

1

produces a changing magnetic field (in the core)

1

this induces an (alternating) potential difference across the secondary (coil)

1

(c)     any two from:

•         if the (local) power station breaks down / fails / demand / load exceeds supply

•         electricity / power can be switched from elsewhere in the system / from other power station(s)

•         electricity can be generated in places remote from customers

•         (in total) fewer power stations are needed

•         power available in rural / remote areas

•         National Grid allows for (better) control of supply and demand

2

[9]