Name:………………………………					Date of Practical:……………………..

Introduction to required practicals

Investigation into the resistance of a resistor

Aim of the experiment: To investigate the relationship between current and voltage for a resistor.

Equipment:

- Power supply

- Variable resistor

- Fixed resistor

- Ammeter

- Voltmeter

- Connecting wires

Method

- Set up the apparatus as shown in the diagram.

- Adjust the variable resistor to give at least six different currents. Take readings of the current and the potential difference across the fixed resistor.

- Repeat three times and take average potential difference.

- Plot a graph of current (on the y axis) against potential difference (on the x axis). Include error bars on potential difference.

- Draw a line of best fit and a line of worst fit.

- Calculate the resistance of the fixed resistor from the gradient of your graph.

- Calculate the error on your resistance by using

| CPAC Criteria | R | O | G | Comments |
| --- | --- | --- | --- | --- |
| 1 (a): Have you correctly followed written instructions? |  |  |  |  |
| 2 (a): Have you used all equipment (including ammeter and voltmeter) correctly? |  |  |  |  |

Table of Results (Landscape or Portrait) CPAC Criteria; R; O; G; Comments. 2 (b): Have you carried out the practical in sequence? Have you made any adjustments for practical issues encountered?. 2 (c): Have you identified control variables and made plans to take account of them? (here, have you controlled temperature of resistor by switching power supply off in between readings?). 2 (d): Have you selected correct measuring equipment and measuring strategies? (sometimes you will be offered a choice of equipment and you’ll have to select what’s most suitable.).

Modifications made to original plan:

Space for calculations and working if needed

CPAC Criteria; R; O; G; Comments. 3 (a): Have you identified hazards and carried out practical safely? (here, have you switched power supply off in between readings to avoid heating of resistor?). 3 (b): Have you used appropriate safety equipment? (no real safety equipment here, but some practicals will require you to carry out a risk assessment and follow it throughout the course of the practical)..

Analysis and final results (Graphs to be done on a separate sheet):

Marking: CPAC Criteria; R; O; G; Comments. 4 (a): Have you made accurate observations? (Here, have you recorded data to correct sig figs. Have you calculated uncertainty in potential difference readings?). 4 (b): Have you recorded results using appropriate units? (is your table clear and accurate with units? Are your sig figs. consistent throughout?).

References:

Marking: CPAC Criteria; R; O; G; Comments. 5 (a): Have you used appropriate software/tools to process data? (Here, have you plotted a graph with sensible scales and labelled axes? Have you calculated gradient, resistance and uncertainty correctly?). 5 (b): Have you cited sources of information? (Sometimes you will compare your results to accepted values. You will also have to plan some practicals yourself. Here, it’s especially important to reference your information.).