Current flows through a

Current

n electric circuit. Current is equal to how much charge is flowing past a point in the circuit every second. A complete circuit is needed for current to flow.

Current is measured in units of amps (A), and an ammeter is used to measure the current at a point in the circuit. The circuit symbol for an ammeter is a circle with an “A” in it.

The charged particles that flow through a circuit are electrons. Electrons are negatively charged particles.

A series circuit only has one path for the current to flow. Therefore, current is the same everywhere in a series circuit.

The diagram to the right shows a series circuit with one lamp. The current is 2 A at each point in the circuit.

A parallel circuit has more than one path for the current to flow. The current going into a junction (that is, where a wire splits or combines) is equal to the current going out of a junction.

The circuit to the left shows a parallel circuit with each path having an identical lamp. Each branch has the same 2 A of current as before. Before and after each junction, though, the current is 4 A. Each path draws additional current from the cell.

To the right is a more complicated circuit diagram. All of the lamps are identical, so there is more resistance on one path than the other. The upper path has the same 2 A of current, but the lower path has twice the resistance (because there are two lamps) and so has half the current (at 1 A).

The overall current drawn from the battery is 3 A. The higher the resistance in a path, the lower the current.

Q1. The following statements are either true or false. State which are true and which are false.

- Current is equal to how much charge is flowing past a point in the circuit every second.

………………………………………………………………………………………………………………………………………………………………………………………………………..

- Protons are the charged particles that flow through a circuit.

………………………………………………………………………………………………………………………………………………………………………………………………………..

- Current can flow through an incomplete circuit.

………………………………………………………………………………………………………………………………………………………………………………………………………..

- In a series circuit, current is different at different points.

………………………………………………………………………………………………………………………………………………………………………………………………………..

Q2. State what we use to measure current in a circuit.

………………………………………………………………………………………………………………………………………………………………………………………………………..

Q3. State the units of current.

………………………………………………………………………………………………………………………………………………………………………………………………………..

Q4. In the space below, draw the circuit symbols for a cell, a bulb and an ammeter.

Q5. For each of the circuits below:

- Circle whether the circuit is series or parallel.

- For any ammeters that do not have a current written by them, write the missing current.