Potential difference

Potential difference

A battery or power supply gives each charge in an electrical circuit energy. Electrical components like lamps or resistors use this energy (which gets transferred into different energy stores). Potential difference is a measure of the difference in energy that charges have between each two points in a circuit.

Potential difference (commonly referred to as voltage) has units of volts and is measured by a voltmeter which goes in parallel across the component you’re measuring the potential difference across. The circuit symbol for a voltmeter is a circle with a “V” in it.

A circuit only experiences a drop in potential difference when the current goes through an electrical component. Therefore, potential difference across each parallel branch is the same.

The diagram to the right shows a cell that produces a potential difference of 10 V across it. The two resistors both have 10 V across them.

In a series circuit, potential difference is split across all components.

In the series circuit to the left, there are two identical resistors. Both resistors have a potential difference of 5 V across them (equal to half the potential difference across the cell).

The circuit diagram to the right is slightly more complicated. The total potential difference across each parallel path is 10 V. However, the lower path has two bulbs and so they each have 5 V across them.

Note how the top lamp is brightest. This is because it has a higher potential difference across it.

Lamps in parallel circuits have two main advantages:

- Each lamp receives the full potential difference and so is brighter.

- If there was a switch on each path, each lamp could be turned on and off independently. Remember that a circuit needs to be complete for a current to flow.

Lamps also have a rating. The lamp rating is the maximum potential difference that is safe to put across a lamp without it breaking.

Q1. The following statements are either true or false. State which are true and which are false.

- The potential difference across each parallel path is the same.

………………………………………………………………………………………………………………………………………………………………………………………………………..

- In a series circuit, potential difference is the same everywhere.

………………………………………………………………………………………………………………………………………………………………………………………………………..

- The higher the potential difference across a lamp, the brighter the lamp.

………………………………………………………………………………………………………………………………………………………………………………………………………..

- The lamp rating is the potential difference that the lamp turns on at.

………………………………………………………………………………………………………………………………………………………………………………………………………..

- Voltmeters go in series in a circuit.

………………………………………………………………………………………………………………………………………………………………………………………………………..

Q2. State an advantage of having lamps in a parallel circuit.

………………………………………………………………………………………………………………………………………………………………………………………………………..

Q3. In the space below, draw the circuit symbols for a resistor and a voltmeter.

Q4. Each of the circuits below have some voltmeters that do not have a current written by them, write the missing potential differences.