Speakers and microphones

A speaker is made from a coil of wire, a permanent magnet and a cone.

A current flows into the coil of wire. Any time a current flows through a wire, a magnetic field is formed around the wire.

Adapted from: Harkonnen2 - Loudspeakerconstruction.png, CC BY-SA 3.0, https://commons.wikimedia.org/w/index.php?curid=9540717

This magnetic field interacts with the field caused by the permanent magnets. This causes a force which, in turn, causes the cone to move.

If the current changes direction, then the magnetic field caused by the coil changes direction. This causes the force on the coil to be in the opposite direction to before.

A current that constantly changes direction is called an alternating current.

If the current constantly changes direction, then so does the direction of the force on the cone. The cone therefore vibrates back and forth, making a sound. The vibration of an object is what makes sound.

The frequency of the sound made is the same as the frequency of vibration. The faster the vibration of the cone, the higher the frequency of sound made.

There are three ways of making this sound louder:

- Increase the current flowing through the coil.

- Increase the number of turns in the coil.

- Increase the strength of the magnetic field caused by the permanent magnet.

A microphone is built in the same way as a speaker, but works in the opposite way.

In a microphone, a sound wave comes onto the cone and causes it to vibrate. In a microphone, the cone is sometimes called a diaphragm.

This causes the coil of wire to vibrate and the coil moves within the magnetic field of the permanent magnet.

Any time a coil of wire experiences a change in magnetic field (as it would when it moves in a magnetic field) then it causes a current to flow in the coil.

This changing current in the coil could then be passed into a speaker for playing (or the pattern of the changing current could be saved to play later).

Q1. The following statements are either true or false. State which are true and which are false.

- Any time a current passes through a wire, a magnetic field is formed around the wire.

………………………………………………………………………………………………………………………………………………………………………………………………………..

- A microphone contains a diaphragm which vibrates when sound reaches it.

………………………………………………………………………………………………………………………………………………………………………………………………………..

- An alternating current is one that always flows in the same direction.

………………………………………………………………………………………………………………………………………………………………………………………………………..

- If a coil moves in a magnetic field, a current is made.

………………………………………………………………………………………………………………………………………………………………………………………………………..

- If the current in the coil in a speaker is decreased, the sound made by the speaker is louder.

………………………………………………………………………………………………………………………………………………………………………………………………………..

Q2. State the names of the three different parts of a speaker.

………………………………………………………………………………………………………………………………………………………………………………………………………..

Q3. Describe what an alternating current is.

………………………………………………………………………………………………………………………………………………………………………………………………………..

Q4. Describe how a speaker makes a sound.

………………………………………………………………………………………………………………………………………………………………………………………………………..

………………………………………………………………………………………………………………………………………………………………………………………………………..

………………………………………………………………………………………………………………………………………………………………………………………………………..

………………………………………………………………………………………………………………………………………………………………………………………………………..

Q5. State three ways of making a speaker make a louder sound.

………………………………………………………………………………………………………………………………………………………………………………………………………..

………………………………………………………………………………………………………………………………………………………………………………………………………..

………………………………………………………………………………………………………………………………………………………………………………………………………..

Q6. Describe how a microphone works.

………………………………………………………………………………………………………………………………………………………………………………………………………..

………………………………………………………………………………………………………………………………………………………………………………………………………..

………………………………………………………………………………………………………………………………………………………………………………………………………..

………………………………………………………………………………………………………………………………………………………………………………………………………..